General Guide to Incompatible Chemicals

The chemicals listed below may undergo a chemical reaction when mixed, these reactions may be violent in nature (producing heat, gas or may even lead to spontaneous ignition) or may produce hazardous, reactive or toxic by-products.  Incompatible chemicals should be segregated wherever possible to prevent them from coming into contact and undergoing a chemical reaction either by mixing or in the event of a leak or spillage.  This list is generic and non-exhaustive and does not cover all possible interactions, for more detailed information always refer to the safety data sheet and other resources.

| Chemical / Class | Incompatible with: | Reason |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Violent Reaction | Flammable Gas | Fire | Unstable | Toxic Product | Toxic Gas |
| Acetic acid | Chromic acid, Chromium (VI) oxide |  |  |  |  |  |  |
|  | Nitric acid |  |  |  |  |  |  |
|  | Alcohols |  |  |  |  |  |  |
|  | Ethylene glycol |  |  |  |  |  |  |
|  | Perchloric acid |  |  |  |  |  |  |
|  | Oxidising agents |  |  |  |  |  |  |
| Acetone | Concentrated nitric / sulphuric acid mixtures |  |  |  |  |  |  |
|  | Strong bases |  |  |  |  |  |  |
|  | Chloroform (in the presence of a base) |  |  |  |  |  |  |
| Acetylene | Copper (including pipes / tubing) |  |  |  |  |  |  |
|  | Chlorine, bromine, fluorine |  |  |  |  |  |  |
|  | Silver, mercury |  |  |  |  |  |  |
| Acids | Inorganic cyanide salts |  |  |  |  |  |  |
|  | Alkali / base |  |  |  |  |  |  |
| Activated Charcoal / Carbon | Calcium hypochlorite |  |  |  |  |  |  |
|  | Oxidising agents |  |  |  |  |  |  |
| Alkali metals (Li, Na, K, Rb, Cs) | Water / Acids |  |  |  |  |  |  |
|  | Carbon tetrachloride / Halogenated alkanes |  |  |  |  |  |  |
|  | Carbon Dioxide (solid)1 |  |  |  |  |  |  |
|  | Halogens |  |  |  |  |  |  |
| Alkaline earth metals (Be, Mg, Ca, Sr, Ba, Ra) | Water / Acids |  |  |  |  |  |  |
|  | Carbon tetrachloride / Halogenated alkanes |  |  |  |  |  |  |
|  | Carbon Dioxide (solid) |  |  |  |  |  |  |
|  | Halogens |  |  |  |  |  |  |
| Ammonia (anhydrous) | Mercury |  |  |  |  |  |  |
|  | Strong oxidisers (e.g. calcium hypochlorite) |  |  |  |  |  |  |
|  | Chlorine, Bromine, Iodine |  |  |  |  |  |  |
|  | Hydrogen fluoride / hydrofluoric acid |  |  |  |  |  |  |
| Ammonia solution (aqueous) | Chlorine, bromine, iodine |  |  |  |  |  |  |
| Ammonium nitrate | Acids |  |  |  |  |  |  |
|  | Metal powders |  |  |  |  |  |  |
|  | Flammable liquids / combustible materials |  |  |  |  |  |  |
|  | Chlorates |  |  |  |  |  |  |
|  | Nitrites |  |  |  |  |  |  |
|  | Sulphur |  |  |  |  |  |  |
| Aniline | Nitric acid |  |  |  |  |  |  |
|  | Hydrogen peroxide |  |  |  |  |  |  |
| Arsenic compounds | Reducing agents |  |  |  |  |  |  |
| Azides (e.g. sodium azide) | Acids |  |  |  |  |  |  |
|  | Lead, copper, heavy metals |  |  |  |  |  |  |
|  | Carbon disulphide |  |  |  |  |  |  |
| Bromine | Ammonia |  |  |  |  |  |  |
|  | Acetylene |  |  |  |  |  |  |
|  | Butadiene |  |  |  |  |  |  |
|  | Flammable / combustible materials |  |  |  |  |  |  |
|  | Sodium carbide |  |  |  |  |  |  |
|  | Turpentine |  |  |  |  |  |  |
|  | Finely divided metals |  |  |  |  |  |  |
| Calcium oxide | Water |  |  |  |  |  |  |
| Chlorate Compounds | Ammonium salts |  |  |  |  |  |  |
|  | Acids2 |  |  |  |  |  |  |
|  | Metal powders |  |  |  |  |  |  |
|  | Sulphur |  |  |  |  |  |  |
|  | Combustible materials |  |  |  |  |  |  |
| Chlorinated solvents (e.g. chloroform) | Alkali metals |  |  |  |  |  |  |
|  | Non-halogenated flammable solvents |  |  |  |  |  |  |
|  | Powdered aluminium or magnesium |  |  |  |  |  |  |
| Chromic acid and chromium (VI) oxide | Acetic acid, acetic anhydride |  |  |  |  |  |  |
|  | Flammable liquids |  |  |  |  |  |  |
| Chlorine | Ammonia |  |  |  |  |  |  |
|  | Hydrogen |  |  |  |  |  |  |
|  | Acetylene |  |  |  |  |  |  |
|  | Methane, ethane, propane, butane |  |  |  |  |  |  |
|  | Alcohols |  |  |  |  |  |  |
|  | Sodium carbide |  |  |  |  |  |  |
|  | Finely divided metals, combustible materials3 |  |  |  |  |  |  |
| Chlorine dioxide | Ammonia |  |  |  |  |  |  |
|  | Methane, ethane, propane, butadiene |  |  |  |  |  |  |
|  | Phosphine |  |  |  |  |  |  |
|  | Hydrogen sulphide |  |  |  |  |  |  |
| Copper | Acetylene |  |  |  |  |  |  |
|  | Hydrogen peroxide |  |  |  |  |  |  |
| Cumene hydroperoxide | Acids |  |  |  |  |  |  |
|  | Combustible materials |  |  |  |  |  |  |
| Cyanide salts | Acids |  |  |  |  |  |  |
| Diethyl ether | Chlorine |  |  |  |  |  |  |
| Dimethyl sulphoxide (DMSO) | Chromium trioxide |  |  |  |  |  |  |
| Ethanol | Silver nitrate |  |  |  |  |  |  |
|  | Calcium hypochlorite |  |  |  |  |  |  |
| Flammable solvents | Oxidising agents |  |  |  |  |  |  |
|  | Reducing agents4 |  |  |  |  |  |  |
|  | Concentrated acids |  |  |  |  |  |  |
| Fluorine | All other chemicals |  |  |  |  |  |  |
| Hydride compounds (e.g. sodium hydride) | Waste chemicals |  |  |  |  |  |  |
| Hydrocarbons (e.g. propane) | Oxidisers (e.g. chlorine, sodium peroxide) |  |  |  |  |  |  |
| Hydrogen cyanide (hydrocyanic acid) | Acids |  |  |  |  |  |  |
|  | Bases / alkalis |  |  |  |  |  |  |
| Hydrofluoric acid | Ammonia |  |  |  |  |  |  |
|  | Bases |  |  |  |  |  |  |
|  | Silica |  |  |  |  |  |  |
| Hydrogen sulphide | Fuming nitric acid |  |  |  |  |  |  |
|  | Acids |  |  |  |  |  |  |
|  | Oxidising gases (e.g. chlorine, oxygen) |  |  |  |  |  |  |
| Hypochlorite compounds | Acids |  |  |  |  |  |  |
|  | Activated carbon |  |  |  |  |  |  |
| Iodine | Acetylene |  |  |  |  |  |  |
|  | Ammonia |  |  |  |  |  |  |
| Mercury | Acetylene |  |  |  |  |  |  |
|  | Fulminic acid |  |  |  |  |  |  |
|  | Ammonia |  |  |  |  |  |  |
| Nitrate compounds | Acids (sulphuric acid) |  |  |  |  |  |  |
| Nitric acid (concentrated) | Acetic acid, acetic anhydride |  |  |  |  |  |  |
|  | Inorganic acids |  |  |  |  |  |  |
|  | Hydrogen sulphide |  |  |  |  |  |  |
|  | Flammable liquids |  |  |  |  |  |  |
|  | Copper, brass |  |  |  |  |  |  |
|  | Heavy metals |  |  |  |  |  |  |
|  | Reducing agents |  |  |  |  |  |  |
| Nitrite compounds | Acids |  |  |  |  |  |  |
| Nitroparaffins (e.g. nitromethane) | Inorganic bases |  |  |  |  |  |  |
|  | Amines |  |  |  |  |  |  |
|  | Silver Nitrate |  |  |  |  |  |  |
| Oxalic acid | Silver, mercury |  |  |  |  |  |  |
| Oxidising agents | Organic materials (e.g. wood, paper) |  |  |  |  |  |  |
|  | Flammable solvents |  |  |  |  |  |  |
|  | Reducing agents |  |  |  |  |  |  |
|  | Dehydrating agents (e.g. sulphuric acid) |  |  |  |  |  |  |
| Oxygen | Oils, grease, flammable liquids, solids or gases |  |  |  |  |  |  |
| Perchloric acid | Acetic acid, acetic anhydride |  |  |  |  |  |  |
|  | Bismuth (and its alloys) |  |  |  |  |  |  |
|  | Combustible materials |  |  |  |  |  |  |
|  | Dehydrating agents (e.g. sulphuric acid) |  |  |  |  |  |  |
| Peroxide compounds (organic) | Acids (organic and mineral)5 |  |  |  |  |  |  |
| Phosphorus (white) | Oxygen / Air |  |  |  |  |  |  |
|  | Oxygen containing compounds (e.g. chlorates) |  |  |  |  |  |  |
|  | Alkalis |  |  |  |  |  |  |
|  | Reducing agents |  |  |  |  |  |  |
|  | Sulphur |  |  |  |  |  |  |
| Phosphorus pentoxide | Water |  |  |  |  |  |  |
| Picric acid | Heavy metal salts (lead, mercury, silver) |  |  |  |  |  |  |
|  | Ammonia |  |  |  |  |  |  |
|  | Alkalis / bases |  |  |  |  |  |  |
| Potassium chlorate / perchlorate | Acids |  |  |  |  |  |  |
|  | Alkali metals |  |  |  |  |  |  |
|  | Magnesium, calcium |  |  |  |  |  |  |
| Potassium permanganate | Glycerol, ethylene, benzaldehyde |  |  |  |  |  |  |
|  | Sulphuric acid |  |  |  |  |  |  |
| Silver | Acetylene |  |  |  |  |  |  |
|  | Oxalic acid, tartaric acid, fulminic acid |  |  |  |  |  |  |
|  | Ammonium compounds |  |  |  |  |  |  |
| Sodium nitrite | Ammonium nitrate |  |  |  |  |  |  |
|  | Acids |  |  |  |  |  |  |
| Sodium peroxide | Alcohols |  |  |  |  |  |  |
|  | Glacial acetic acid, acetic anhydride |  |  |  |  |  |  |
|  | Benzaldehyde |  |  |  |  |  |  |
|  | Carbon disulphide |  |  |  |  |  |  |
|  | Organic compounds (e.g. ethylene glycol) |  |  |  |  |  |  |
| Sulphide compounds | Acids |  |  |  |  |  |  |
| Sulphuric acid | Potassium chlorate / perchlorate |  |  |  |  |  |  |
|  | Potassium permanganate (or similar) |  |  |  |  |  |  |
|  | Metals |  |  |  |  |  |  |
|  | Hypochlorite compounds |  |  |  |  |  |  |

Colour Key:

Violent / Exothermic Reaction

Flammable gas generated

Fire / Spontaneous ignition

Unstable / Explosive Compound Formed

Toxic substance produced

Toxic gas generated

Notes:

- Carbon dioxide fire extinguishers should not be used on alkali metal fires as they will tend to increase the severity of the fire.

- Chlorine dioxide (an explosive gas) is a by-product of this reaction

- Explosive mixtures formed with turpentine, diethylether and many other organic liquids

- For example lithium aluminium hydride will react vigorously with protic solvents such as alcohols

- Avoid friction and store at reduced temperature